BITS PILANI DISSERTATION / PROJECT OUTLINE REPORT
Post-Quantum Cryptography (PQC) Integration in Embedded Secure Bootloaders (MCUboot, U-Boot FIT, and EDKII/UEFI)

| Course Title: | Dissertation / Project Work |
| --- | --- |
| Degree Program: | M.Tech in Embedded Systems / Cyber Security |
| Research Area: | Embedded Security, Post-Quantum Cryptography, Firmware Trust Anchors, QEMU Emulation |
| Institution: | Birla Institute of Technology & Science, Pilani (Vidya Vihar, Pilani, Rajasthan - 333031) |
| Target Bootloaders: | MCUboot (ARM Cortex-M4), U-Boot FIT (RISC-V 64), EDKII / UEFI SecurityPkg (ARM Cortex-A57 4-Core SMP) |
| PQC Algorithms: | NIST FIPS 204 ML-DSA-44, NIST FIPS 205 SPHINCS+, RFC 8554 LMS / LMOTS |

## 1. Broad Area of Work

This project operates at the intersection of Embedded Systems Security, Post-Quantum Cryptography (PQC), Firmware Root of Trust (RoT) Architecture, and Hardware Emulation. The broad research area focuses on modernizing Stage 0 and Stage 1 secure bootloaders to withstand quantum cryptanalytic attacks using NIST-standardized algorithms (ML-DSA-44, SPHINCS+, LMS) under strict zero-malloc memory bounds.

## 2. Background

Classic secure bootloaders rely on RSA-2048/4096 and ECDSA P-256 digital signatures to verify firmware integrity before execution. Shor's Algorithm running on a quantum computer solves integer factorization and discrete logarithms in polynomial time O((log N)^3), completely breaking RSA and ECDSA. Integrating post-quantum cryptography into embedded bootloaders requires overcoming memory constraints (PQC keys/signatures are 10-30x larger), strict zero-dynamic-memory rules (<4KB SRAM), and container diversity (MCUboot TLVs, U-Boot FIT Device Trees, UEFI PE/COFF WIN_CERTIFICATE headers).

## 3. Objectives

1. Design & build a zero-malloc C99 PQC verification engine (pqc_crypto.c) for ML-DSA-44, SPHINCS+, and LMS.
2. Extend MCUboot (ARM Cortex-M4) with PQC TLVs (0x80-0x88) and imgtool signing utilities.
3. Integrate U-Boot FIT (RISC-V 64) with U_BOOT_CRYPTO_ALGO(ml_dsa_44) linker tables and mkimage.
4. Integrate EDKII / UEFI SecurityPkg (ARM Cortex-A57 4-Core SMP) with PKCS#7 PQC OID verifiers.
5. Conduct a 5-subagent peer audit consensus (100% approval) and build an automated test harness (23/23 tests passed).
6. Scaffold a Docusaurus documentation web platform with interactive React visual playgrounds and GDB debugging tutorials.

## 4. Scope of Work

The scope encompasses full-stack implementation across 3 major bootloaders, automated test suite verification, QEMU emulator launchers for 3 CPU architectures, GDB multi-arch debugging tutorials, multi-agent audit reporting, and a production Docusaurus web portal.

## 5. Plan of Work

Phase 1: Architecture & MCUboot Integration (Cortex-M4, imgtool, TLVs)
Phase 2: U-Boot FIT Image Integration (RISC-V 64, mkimage, linker tables)
Phase 3: EDKII / UEFI SecurityPkg Integration (ARM Cortex-A57 4-Core SMP, PKCS#7 OIDs)
Phase 4: Multi-Agent Audit & Edge-Case Verification (23/23 tests passing)
Phase 5: Docusaurus Documentation Web Portal & Interactive React Playgrounds

## 6. Literature References

1. NIST FIPS 204: Module-Lattice-Based Digital Signature Standard (ML-DSA), Aug 2024.
2. NIST FIPS 205: Stateless Hash-Based Digital Signature Standard (SLH-DSA), Aug 2024.
3. RFC 8554: Leighton-Micali Hash-Based Signatures (LMS/LMOTS), IETF, 2019.
4. RFC 8708: Use of LMS Algorithm in Cryptographic Message Syntax (CMS), IETF, 2020.
5. Shor, P. W.: Algorithms for quantum computation: discrete logarithms and factoring, IEEE, 1994.
6. MCUboot Architecture Specification (https://mcu-tools.github.io/mcuboot/)
7. U-Boot FIT Signature Verification Specification (https://docs.u-boot.org/)
8. UEFI Specification Version 2.10, UEFI Forum, 2023.